AJAX Device Configuration Guide – GCX-ONE

Overview:

GCX-ONE supports AJAX integration for centralized alarm reception. AJAX hubs containing NVRs, intrusion panels, and other alarm-triggering objects can be integrated with GCX-ONE for centralized alarm management.

Key Features of AJAX Integration:

1. Supported Device Types

  - AJAX Hub

  - AJAX NVR

2. Supported Functionality

  - Discovery

  - Alarm reception only
 (No live streaming, playback, or PTZ features)

3. Alarm Transmission Protocol

  - SIA (Security Industry Association Protocol)

How AJAX Works in GCX-ONE:

AJAX Hub / AJAX NVR:

- The AJAX Hub acts as a container for multiple alarm objects such as intrusion panels, NVRs, and other sensors.

- In GCX-ONE, you can add either:

- AJAX Hub (AJAX manufactured cameras)

- AJAX NVR (Third party cameras)

- All alarms from Hub/NVR objects are received collectively under this device in GCX-ONE.

Steps to Configure AJAX Device in GCX-ONE:

1. Obtain Hub ID from AJAX Hub or Device ID for AJAX NVR

- Hub ID is required for adding AJAX Hub and Device ID for NVR.

- The Hub ID/Device ID must be collected from the AJAX client application (specific to the customer's Hub).

Important Note:

When entering the Hub ID or Device ID:

- GCX-ONE performs an existence check using the NXGEN API key.

- To allow verification, the customer must invite NXGEN to their AJAX Hub through the AJAX client application. Without this invitation, GCX-ONE cannot verify or link to the Hub/NVR.

2. Add AJAX Device in GCX-ONE

Step 1: Log in to GCX-ONE.

Step 2: Click on the Configuration menu.

Step 3: At the Service Provider level, click on Customers.

Step 4: Click the Add button.

Step 5: Enter the required fields and click Save.

Step 6: Select the newly created customer.

Step 7: Navigate to the Sites tab.

Step 8: Click the Add button.

Step 9: Enter the required fields and click Save.

Step 10: Select the newly created site.

Step 11: Navigate to the Devices tab.

Step 12: Click the Add button.

Step 13: The Add Device dialog will be displayed.

Step 14: In the dialog, open the Device dropdown and select AJAX Hub or AJAX NVR.

### Step 15: Enter all required fields:

### Name

### Time Zone

- Hub ID (for AJAX hub)

- Device ID (for AJAX NVR)

Step 16: Click Discover.

- The platform will connect to and verify the Hub/NVR.

- On success:

- For Hub → All devices associated with the Hub will be automatically fetched and displayed in a list.

- For NVR → All cameras associated with the NVR will be automatically fetched and displayed in a list.

- Review the discovered devices or cameras.

- Confirm the details.

Step 17: Once the device is successfully discovered, click Save.

3. Configure SIA Alarm Receiver (AJAX Client):

- After adding the device:

- Locate your newly registered AJAX device in the device list.

- Click the View button beside the device.

- On the device overview page, The required information will be displayed as shown in the screen below:

4. Steps in AJAX Client:

Receiver IP/PORT: Configure at Company Level in the AJAX client.

Encryption Key: Configure at Company Level in the AJAX client.

Account Number: Configure at Object Level in the AJAX client (for the Hub or relevant alarm object).

Refer to the following guide for detailed receiver configuration instructions:

AJAX Integration Guide

Limitations